# IoT Smart Home Controller

User Manual

Version 1.0

December 2025

Sheridan College

Computer Engineering Technology

# Table of Contents

1. Introduction

2. Safety Information

3. Package Contents

4. Hardware Setup

4.1 Physical Installation

4.2 Wiring Connections

4.3 Sensor Placement

5. Software Configuration

5.1 WiFi Setup

5.2 MQTT Broker Configuration

6. Android App Installation

7. Using the System

7.1 Dashboard Overview

7.2 Monitoring Temperature & Humidity

7.3 Controlling Relays

8. Troubleshooting

9. Maintenance

10. Technical Specifications

11. Support & Contact

# 1. Introduction

Thank you for choosing the IoT Smart Home Controller. This system allows you to monitor environmental conditions and control household appliances from anywhere using your Android smartphone.

### Key Features

- Real-time temperature and humidity monitoring

- 4-channel relay control for appliances

- Remote access via mobile application

- Low-latency response (<500ms)

- Secure cloud communication via MQTT

# 2. Safety Information

⚠ WARNING - ELECTRICAL SAFETY This device controls high-voltage electrical circuits. Improper installation or use can result in electrical shock, fire, or death. If you are not qualified to work with electrical systems, please consult a licensed electrician.

## 2.1 General Safety Guidelines

- Disconnect power before performing any installation or maintenance

- Do not exceed the maximum load rating of 10A per relay channel

- Ensure proper grounding of all electrical connections

- Keep away from water and moisture

- Install in ventilated area to prevent overheating

- Do not modify the device hardware without proper knowledge

# 3. Package Contents

Please verify that your package contains all the following items:

| Item | Quantity | Check |
| --- | --- | --- |
| Smart Home Controller Board (PCB) | 1 | ☐ |
| ESP32 DevKit Module | 1 | ☐ |
| DHT22 Temperature/Humidity Sensor | 1 | ☐ |
| 4-Channel Relay Module | 1 | ☐ |
| USB Power Cable | 1 | ☐ |
| Quick Start Guide | 1 | ☐ |

# 4. Hardware Setup

## 4.1 Physical Installation

Choose an installation location that meets the following criteria:

- Protected from direct water exposure

- Within range of your WiFi network

- Accessible for maintenance

- Near electrical connections for controlled devices

## 4.2 Wiring Connections

### Step-by-Step Wiring Guide

- Power off all circuits you will be connecting to the relay module

- Insert the ESP32 module into the PCB header socket, ensuring correct orientation

- Connect the DHT22 sensor to the designated 3-pin header (VCC, DATA, GND)

- Connect the relay module control pins to the GPIO headers

- Wire your appliances through the relay contacts (COM and NO terminals)

- Connect USB power to the ESP32 module

### Pin Mapping Reference

| Component | ESP32 GPIO | Wire Color |
| --- | --- | --- |
| DHT22 Data | GPIO4 | Yellow |
| Relay 1 | GPIO16 | Blue |
| Relay 2 | GPIO17 | Green |
| Relay 3 | GPIO18 | Orange |
| Relay 4 | GPIO19 | Purple |

## 4.3 Sensor Placement

For accurate readings, place the DHT22 sensor:

- Away from direct sunlight or heat sources

- At a representative height (approximately 1.5m from floor)

- With adequate air circulation

- Away from air conditioning vents or drafts

# 5. Software Configuration

## 5.1 WiFi Setup

The ESP32 must be configured with your WiFi network credentials. This can be done through the firmware configuration file or via serial connection.

### Configuration Parameters

WIFI_SSID = Your network name
WIFI_PASSWORD = Your network password

ℹ NOTE: The system requires a 2.4GHz WiFi network. 5GHz networks are not supported by the ESP32.

## 5.2 MQTT Broker Configuration

The system uses HiveMQ Cloud as the MQTT broker. Default configuration:

| Parameter | Value |
| --- | --- |
| Broker Address | *.hivemq.cloud |
| Port | 8883 (TLS) |
| Security | TLS/SSL Enabled |

# 6. Android App Installation

## 6.1 Installation Steps

- Download the APK file from the project repository or scan the QR code provided

- Enable "Install from Unknown Sources" in your Android settings

- Open the APK file and tap "Install"

- Launch the "IoT Home Controller" app

## 6.2 Initial Configuration

- Open the app and navigate to Settings

- Enter your MQTT broker credentials:

- Broker URL

- Username

- Password

- Tap "Connect" to establish connection

- Verify the connection status indicator turns green

# 7. Using the System

## 7.1 Dashboard Overview

The main dashboard displays all sensor readings and relay controls in a single view:

- Top Section: Connection status indicator and settings access

- Middle Section: Real-time temperature and humidity display

- Bottom Section: Relay control switches (4 channels)

## 7.2 Monitoring Temperature & Humidity

Sensor readings update automatically every 5 seconds. The display shows:

- Temperature: Displayed in Celsius (°C) with ±0.5°C accuracy

- Humidity: Displayed as relative humidity (%) with ±2-5% accuracy

## 7.3 Controlling Relays

Each relay channel can be individually controlled:

- Locate the relay you want to control (Relay 1-4)

- Tap the toggle switch to change state

- Wait for confirmation (switch color change and LED indicator on device)

- The connected appliance will turn ON/OFF accordingly

| Relay | ON State | OFF State |
| --- | --- | --- |
| Relay 1-4 | Green indicator, Circuit closed | Gray indicator, Circuit open |

# 8. Troubleshooting

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| ESP32 not powering on | USB cable issue or no power | Try different USB cable; check power adapter |
| WiFi not connecting | Wrong credentials or 5GHz network | Verify SSID/password; use 2.4GHz network |
| App shows 'Disconnected' | MQTT broker unreachable | Check internet; verify broker settings |
| Sensor shows 'NaN' or error | DHT22 wiring or connection | Check sensor connections; verify GPIO pin |
| Relay not responding | Wiring issue or relay fault | Check relay module connections; test with LED |
| Slow response time | Network congestion | Move closer to WiFi router; reduce network load |

## 8.1 LED Status Indicators

- Solid Blue: System running normally, WiFi connected

- Blinking Blue: Attempting WiFi connection

- Red: Error state - check connections

- Off: No power to device

# 9. Maintenance

## 9.1 Regular Maintenance

- Inspect wiring connections monthly for signs of wear or loosening

- Clean the DHT22 sensor with compressed air to prevent dust buildup

- Check relay contacts for any signs of arcing or damage

- Verify firmware is up to date periodically

## 9.2 Firmware Updates

Firmware updates may be released to add features or fix issues. Check the project repository for the latest version and follow the update instructions provided.

# 10. Technical Specifications

| Parameter | Specification |
| --- | --- |
| Microcontroller |  |
| Model | ESP32-WROOM-32 |
| CPU | Dual-core Xtensa LX6 @ 240MHz |
| Wireless | WiFi 802.11 b/g/n (2.4GHz) |
| Sensor (DHT22) |  |
| Temperature Range | -40°C to 80°C |
| Temperature Accuracy | ±0.5°C |
| Humidity Range | 0-100% RH |
| Humidity Accuracy | ±2-5% RH |
| Relay Module |  |
| Channels | 4 |
| Maximum Load | 10A @ 250VAC / 10A @ 30VDC |
| Communication |  |
| Protocol | MQTT v3.1.1 |
| Latency | <500ms round-trip |
| Power |  |
| Input Voltage | 5V DC (USB) |
| Current Consumption | ~500mA (typical) |

# 11. Support & Contact

## 11.1 Project Resources

- GitHub Repository: https://github.com/Aryan-1912/CAPSTONE_HOUSEHOLD_AUTOMATION_IOT

- Demo Video: Available in repository (DEMO_VIDEO.mp4)

- PCB Design Files: IOT.fsch, IOT.fbrd (Autodesk Fusion 360 Eagle)

## 11.2 Project Team

Lead Developer: Aryan Prajapati

Team Members: Manav Parikh, Dev Patel

Institution: Sheridan College, Computer Engineering Technology

## 11.3 Reporting Issues

If you encounter any issues with the system, please report them through the GitHub Issues page with the following information:

- Description of the problem

- Steps to reproduce the issue

- Expected behavior vs. actual behavior

- Screenshots or logs if available

- Hardware and software versions

— End of User Manual —

Version 1.0 | December 2025